# M5Stack Tab5 + INA3221 Current Sensor

by Bryan A. “CrazyUncleBurton” Thompson

Last Updated 2/21/2026

## Hardware

### Microcontroller Dev Board

M5Stack Tab5

### Sensor

Adafruit INA3221 Triple Current / Voltage Sensor

### Connections

- Connect the USB C Cable – USB C on the Tab5 end and whatever type will plug into your computer on the other end.

- Connect the 4 pin plug end of the sensor cable to red port on M5Stack Tab5.

- Connect the 4 wire Grove to Dupont connector cable to the pins on the INA3221 sensor:

| Sensor | Cable/Port |
| --- | --- |
| VCC | Red/VCC |
| GND | GND |
| SDA | Yellow/SDA |
| SCL | White/SCL |

## Testing

- Connect a USB cable from the computer that VS Code is running on to the M5Stack microcontroller.

- If the microcontroller is not on, press the white button on the side once to turn it on.

- In VS Code, click the Platform IO Upload icon  in the bottom toolbar.  The hover text will be Platform IO: Upload.

- The project will compile.  When it’s ready to upload, we see lines like “Connecting…” and “Writing…”.  Wait for the line that says SUCCESS in green text before proceeding.

- When finished, the microcontroller will reboot.

- When the microcontroller reboots, watch the display.  You should see a one-time splash screen that says M5Stack Tab5 Starting… and then INA3221 Found

- After about one second (a delay inserted to let the user see the splash screen), we see the first update from the current and voltage sensors.

- One second later (again, to read the screen), it refreshes the data and updates the screen.

## Program Explanation

We started with the program from the manufacturer as an example for using their sensor.  It only output data to the LCD.

We added two libraries – one provided by the Microcontroller manufacturer and one provided by the Sensor manufacturer:

#include <M5Unified.h>

#include "Adafruit_INA3221.h"

We added some variables to store text size and formatting variables:

int lineHeight = 50;

int x_margin = 25;

int y_header = 25;

int x = x_margin;

int y = y_header;

We added some constants to store I2C pins:

const int i2cSdaPin = 53; - Wire() is connected to the external bus on these pins

const int i2cSclPin = 54; - Wire() is connected to the external bus on these pins

We create an instance / object of the Adafruit_INA3221 library called ina3221:

Adafruit_INA3221 ina3221;

We create an instance / object of the M5GFX video driver library (part of M5Unified library) with the name “display”:

M5GFX& display = M5.Display;

We create an instance / object of the M5Canvas library (part of the M5Unified library)  this line to create a canvas named “canvas”:

M5Canvas canvas(&display);

The setup() function runs once at microcontroller power-up and restart.

These lines start the M5Unified library which supports the microcontroller:

auto cfg = M5.config();

M5.begin(cfg);

This line starts the Arduino Wire(), which is the external I2C bus where the sensor is connected:

Wire.begin(i2cSda, i2cScl);

These lines configure the display and canvas:

display.setRotation(1);   - This sets which way is up on the LCD.

canvas.setColorDepth(16); - This sets the number of bits of color info on the LCD.

canvas.createSprite(display.width(), display.height()); - These lines create the canvas, which is the background area where we build the screen. It won’t be displayed until later.

canvas.setTextColor(TFT_WHITE, TFT_BLACK); - This sets text color to white, and black background color to white.

canvas.setFont(&fonts::efontJA_16_b); - This is a built-in font that supports Unicode symbols

canvas.setTextSize(textSize); - This sets the size of text we are printing to the screen.

canvas.setTextWrap(false, false); - This says not to wrap the text if it prints off the end of the line

These lines draw the initial splash screen:

canvas.fillScreen(TFT_BLACK); - clear the canvas

y=200; - text position for the splash screen chosen to be approx. centered on the screen

x=475;

canvas.setCursor(x, y); - start writing to the canvas at the new coordinates

canvas.printf(" M5Stack Tab5");

y=275; - Update y to the next row

canvas.setCursor(x, y); - start writing to the canvas at the new coordinates

canvas.printf("  Starting...");

if (!ina3221.begin(0x40, &Wire)) { - Initialize the library and test if the sensor is connected - the sensor’s address on the I2C bus is 0x40 and it is on the external I2c BUS

y=425;

canvas.setCursor(x, y);

canvas.printf("Failed to find INA3221 chip");  - if we get to here, we didn’t find the sensor

canvas.pushSprite(0, 0);

while (1)

delay(10); - wait 10mS and try the sensor again

}

y=425;

canvas.setCursor(x, y);

canvas.printf("INA3221 Found!"); - if we get to here, we found the sensor

y += lineHeight;

These lines configure the operating mode of the sensor:

ina3221.setAveragingMode(INA3221_AVG_16_SAMPLES); - smooth data from the sensor

for (uint8_t i = 0; i < 3; i++) {

ina3221.setShuntResistance(i, 0.05); - tell the sensor about our shunt

}

These lines publish the canvas to the LCD:

canvas.pushSprite(0, 0); - send the canvas to the LCD

delay(3000); - wait to let the user read the screen

This is the end of the one-time setup() function.

The function drawDashboard() creates the dynamic canvas with sensor data:

y = y_header; - space above the first row.

x = x_margin; - margin to the left of text

canvas.setCursor(x, y); - move cursor to top left position

canvas.fillScreen(TFT_BLACK); - clear screen

canvas.setTextSize(textSize); - set size of text to write

for (uint8_t i = 0; i < 3; i++) {  -  loop to read all the sensor data as long as i < 3.  When i=3, stop looping and proceed to the next instruction after the loop.

float voltage = ina3221.getBusVoltage(i); - reads sensor voltage on channel i and assign to floating variable voltage

float current = ina3221.getCurrentAmps(i) * 1000; - reads sensor current on channel i and convert to mA and assign to floating variable current.

canvas.setCursor(x, y); - move the cursor to the next line

canvas.printf("Channel %d Voltage = %.2fV Current = %.2fmA", i, voltage, current); - This line writes a line to the canvas containing the following:

- “Channel“ and then the contents of the “i” variable which is the loop index and contains the current channel we’re reading, formatted as an integer.

- “ Voltage =” and then the voltage measurement, formatted as a floating point with two digits after the decimal, and then “V” (the units of our voltage measurement)

- “Current = “ and then the current measurement, formatted as a floating point with two digits after the decimal, and then “mA” (the units of our current measurement)

y += lineHeight; // before we loop, update y to write to the next line on the LCD

}

That’s the end of our loop.  Now let’s update the screen:

canvas.pushSprite(0, 0); // Once we gather the data and create the canvas, make the data live.

y=y_header; // Reset y to the start of the screen for next update

Now we’re running in the main() loop:

This code causes the main loop to run once every second:

static uint32_t lastFrameMs = 0;

const uint32_t frameIntervalMs = 1000;

if (millis() - lastFrameMs < frameIntervalMs) {

return;

}

lastFrameMs = millis();

This calls the function which creates the canvas and then publishes the canvas:

drawDashboard();

When we get to the end of the list, we loop back to the start of the main loop and run it again.  The program doesn’t end.

## Reference

Tab5 Microcontroller Info:  https://docs.m5stack.com/en/core/Tab5

Sensor Info:  https://learn.adafruit.com/adafruit-ina3221-breakout

M5GFX Display Library Stuff: https://docs.m5stack.com/en/arduino/m5gfx/m5gfx_functions

M5Canvas Stuff:  https://docs.m5stack.com/en/arduino/m5gfx/m5gfx_canvas